# Rocky Linux 9.5 - sedutil and NVMe CLI Installation Procedure

Purpose: Provide a repeatable procedure for installing sedutil-cli and nvme-cli on a fresh Rocky Linux 9.5 installation for TCG Opal / SED NVMe drive management.

WARNING: PSID Revert, SID Revert, initialSetup, and NVMe firmware download/commit operations can modify drive security state or make data inaccessible. This document first installs and verifies the required tools; destructive operations should only be performed against the verified target drive under an approved procedure.

# 1. Verify Rocky Linux Version

Confirm that the fresh installation is Rocky Linux 9.5 and record the system architecture.

cat /etc/rocky-release
uname -m

Expected OS result: Rocky Linux release 9.5 (Blue Onyx). Typical x86-64 architecture result: x86_64.

# 2. Confirm Network Connectivity

Internet access is required to install Rocky packages and clone the sedutil source repository.

ping -c 4 github.com

# 3. Update Rocky Linux

Refresh repository metadata and install current updates.

sudo dnf update -y

If a kernel or major system update is installed, reboot before continuing.

sudo reboot

# 4. Install NVMe CLI

Install nvme-cli, which is used for NVMe identification, firmware download, firmware commit, and firmware verification.

sudo dnf install -y nvme-cli

nvme version
sudo nvme list

Verify that the target NVMe drive appears in the device list.

# 5. Install sedutil Build Prerequisites

Install the compiler and GNU Autotools components required to build sedutil from source.

sudo dnf install -y git gcc gcc-c++ make autoconf automake libtool m4

git --version
gcc --version
g++ --version
make --version
autoconf --version
automake --version

# 6. Create the Source Directory

Use /usr/local/src as the repeatable location for locally built source software.

sudo mkdir -p /usr/local/src
sudo chown "$USER":"$USER" /usr/local/src
cd /usr/local/src

# 7. Download sedutil Source

Clone the Drive Trust Alliance sedutil repository and enter the source directory.

git clone https://github.com/Drive-Trust-Alliance/sedutil.git
cd sedutil

ls

# 8. Record the sedutil Revision

Record the exact source revision used for the build. This is important for repeatability because the repository can change over time.

git rev-parse HEAD
git branch --show-current

For a controlled build, replace the moving branch with a validated commit and check out that exact commit on every fresh installation:

git checkout <VALIDATED_COMMIT_HASH>

# 9. Generate the Build Files

autoreconf --install

Confirm that the configure script was created:

ls -l configure

# 10. Configure sedutil

./configure

The command should complete without a fatal error and create the Makefile/configuration files.

# 11. Compile sedutil

make -j"$(nproc)"

The build must complete without fatal compilation errors.

# 12. Locate the sedutil-cli Binary

find . -type f -name 'sedutil-cli' -perm /111

On the standard upstream build, the executable should be available as ./sedutil-cli.

# 13. Test the Compiled Binary

Before installing it system-wide, verify that the compiled executable can enumerate storage devices.

sudo ./sedutil-cli --scan

# 14. Install sedutil-cli System-Wide

sudo cp ./sedutil-cli /usr/local/sbin/sedutil-cli
sudo chmod 755 /usr/local/sbin/sedutil-cli

command -v sedutil-cli

Expected result:

/usr/local/sbin/sedutil-cli

sudo sedutil-cli --scan

# 15. Identify the Exact NVMe Device

Never assume the target is /dev/nvme0. Identify the drive by model and serial number before any TCG or firmware operation.

sudo nvme list
lsblk -o NAME,MODEL,SERIAL,SIZE,TYPE

A namespace may appear as /dev/nvme0n1 while controller-level operations typically use /dev/nvme0. On systems with multiple NVMe devices, enumeration can differ between systems or boots.

# 16. Query the Target Drive with sedutil

Replace /dev/nvme0 with the verified target NVMe controller.

sudo sedutil-cli --query /dev/nvme0

The firmware/TCG procedure supplied for this work uses LockingEnabled as a state check: LockingEnabled = N indicates the manufactured-inactive/reverted state, while LockingEnabled = Y is expected after TCG activation.

# 17. Record the Current NVMe Firmware Revision

sudo nvme list
sudo nvme id-ctrl /dev/nvme0

To display the firmware revision field directly:

sudo nvme id-ctrl /dev/nvme0 | grep -i '^fr'

Record the current firmware revision before performing an upgrade.

# 18. Record Installation Versions

For a repeatable engineering or production process, capture the following information in the build/test record.

cat /etc/rocky-release
uname -r
nvme version
cd /usr/local/src/sedutil && git rev-parse HEAD
gcc --version | head -1

# Installation Verification Checklist

☐ Rocky Linux reports version 9.5.

☐ System architecture is recorded.

☐ Rocky package repositories / network access are functional.

☐ nvme-cli is installed and nvme version executes.

☐ sudo nvme list detects the target drive.

☐ Git and required development tools are installed.

☐ sedutil source was cloned from the Drive Trust Alliance repository.

☐ The exact sedutil Git commit was recorded.

☐ autoreconf --install completed successfully.

☐ ./configure completed successfully.

☐ make -j"$(nproc)" completed successfully.

☐ sedutil-cli was compiled and copied to /usr/local/sbin.

☐ command -v sedutil-cli returns /usr/local/sbin/sedutil-cli.

☐ sudo sedutil-cli --scan executes successfully.

☐ Target NVMe drive model and serial number were verified.

☐ sudo sedutil-cli --query /dev/nvmeX successfully queries the target drive.

☐ Current NVMe firmware revision was recorded.

# Reference Commands for the Firmware/TCG Procedure

The supplied firmware procedure uses sedutil for TCG revert/query/activation and nvme-cli for firmware download, commit, and verification. The following commands are references only; verify the target device, firmware image, credentials, and approved procedure before use.

sudo sedutil-cli --scan
sudo sedutil-cli --query /dev/nvme0

sudo nvme fw-download /dev/nvme0 -f file.bin
sudo nvme fw-commit /dev/nvme0 -s 0 -a 3
sudo nvme list

# NVMe vs. SATA Note

This procedure is intended for NVMe drives. The Linux libata.allow_tpm=1 setting sometimes referenced in sedutil documentation applies to SATA-based Opal drives and is not required for NVMe devices.

# Installation Stop Point

The software installation is complete when both of the following execute successfully:

sudo sedutil-cli --scan
sudo nvme list

Do not proceed to PSID Revert, SID Revert, --initialSetup, or firmware download/commit until the target drive and the approved operational procedure have been verified.

# Source Procedure Alignment

The provided Firmware Upgrade & TCG Configuration Procedure specifies a TCG revert before the firmware upgrade, verification of LockingEnabled = N, NVMe firmware download/commit, firmware-version verification, TCG activation, verification of LockingEnabled = Y, and a final revert/check. This installation document prepares a fresh Rocky Linux 9.5 system with the tools required for those operations.